# SPD FEASIBILITY REPORT

Bliss Palm Bay | 2165 Sandy Pines Dr NE

# Executive Summary

Project: Bliss Palm Bay (SPD-2025-001)

Address: 2165 Sandy Pines Dr NE, Palm Bay, FL 32907

Account ID: 2835546 | Legal: S27, T28S, R37E

Parcel Area: 46,394 SF (1.065 acres)

Proposed Use: 1-2 Buildings, 3-4 Stories Multifamily

ML FEASIBILITY SCORE: 63.3 / 100 (Grade: C+)

Recommendation: PROCEED WITH CAUTION - Marginally feasible. Success depends on cost control and market conditions.

# 12-Stage Pipeline Summary

| # | Stage | Status | Key Output |
| --- | --- | --- | --- |
| 1 | Discovery | ✓ | Property profile, contacts |
| 2 | Survey Analysis | ✓ | Boundaries, easements mapped |
| 3 | Zoning Analysis | ✓ | RM-20 likely required |
| 4 | Site Constraints | ✓ | Scrub-jay/tortoise assessed |
| 5 | Parking Analysis | ✓ | 48 spaces for 24 units |
| 6 | Traffic Impact | ✓ | 162 daily trips - low impact |
| 7 | Utilities | ✓ | City water/sewer available |
| 8 | Stormwater | ✓ | SJRWMD ERP required |
| 9 | Site Layout | ✓ | Conceptual layout complete |
| 10 | ML Feasibility | ✓ | Score: 63.3 (C+) |
| 11 | Report | ✓ | This document |
| 12 | Archive | ✓ | GitHub + Supabase |

# Key Findings

## Survey Analysis

- Frontage: 251.61' arc on Sandy Pines Drive

- Tract O utility easements (NW and east side)

- FPL aerial easement on west side - OK for parking

## Site Constraints

- Scrub-Jay: Palm Bay has 30-year ITP - $125/quarter acre fee

- Gopher Tortoise: Survey required - HIGH risk in sandy soils

- Flood Zone: Verify on Palm Bay GIS before proceeding

## Parking & Traffic

- 24 units @ 2.0 spaces/unit = 48 spaces required

- Traffic: 162 daily trips - No full study required

- West side parking area can accommodate ~38 spaces surface

# Financial Analysis (24-Unit Scenario)

| Metric | Value |
| --- | --- |
| Total Development Cost | $4,920,000 |
| Cost Per Unit | $205,000 |
| Gross Potential Rent (Annual) | $463,200 |
| Net Operating Income | $286,026 |
| Cap Rate | 5.81% |
| Stabilized Value (6% Cap) | $4,767,100 |

# Priority Action Items

- HIGH: Schedule pre-application meeting with Palm Bay Land Development (321) 733-3042

- HIGH: Verify flood zone and zoning on Palm Bay GIS

- HIGH: Conduct gopher tortoise survey before any site clearing

- MEDIUM: Get construction cost estimates from 2-3 general contractors

- MEDIUM: Explore affordable housing incentives to improve returns

# Key Contacts

| Palm Bay Land Development | (321) 733-3042 |
| --- | --- |
| Palm Bay Building | (321) 953-8924 |
| Palm Bay Utilities | (321) 952-3420 |
| FWC (Gopher Tortoise) | (321) 383-5535 |
| SJRWMD (Stormwater) | (386) 329-4500 |

Report Generated: December 9, 2025

SPD Site Plan Development Agentic AI Pipeline v1.0